Test sul robot reale: run 1 vs run 2 (ANOVA a misure ripetute, p corretto Holm ×2 per metrica)

| Metrica | Fase | F(1, 9) | p Holm | ηp² |
| --- | --- | --- | --- | --- |
| Completion time (log) | Andata | 20.30 | 0.002 ** | 0.69 |
|  | Ritorno | 20.90 | 0.002 ** | 0.70 |
| Path length | Andata | 1.27 | 0.289 | 0.12 |
|  | Ritorno | 5.13 | 0.100 | 0.36 |
| Region transitions | Andata | 0.40 | 0.542 | 0.04 |
|  | Ritorno | 3.98 | 0.154 | 0.31 |
| Mean speed | Andata | 29.20 | < 0.001 *** | 0.76 |
|  | Ritorno | 40.00 | < 0.001 *** | 0.82 |

Statistical analysis

Performance in the two tests on the real robot was compared separately for the outbound and return navigation phases. For each metric, a repeated-measures ANOVA was performed with Test (Test1, Test2) as within-subject factor. Completion time was log-transformed to meet the normality assumption, while path length, number of region transitions and mean speed were analyzed on the original scale. The Holm correction was applied to account for the two comparisons (outbound and return) performed for each metric.

Results

Navigation with the real robot

Participants completed the navigation faster in the second test on the real robot (figure X). Completion time decreased significantly from the first to the second test in both the outbound (F(1, 9) = 20.30, p = 0.002, ηp² = 0.69) and the return phase (F(1, 9) = 20.90, p = 0.002, ηp² = 0.70). Consistently, the mean speed of the robot increased in both phases (outbound: F(1, 9) = 29.20, p < 0.001, ηp² = 0.76; return: F(1, 9) = 40.00, p < 0.001, ηp² = 0.82).

In contrast, the distance traveled by the robot did not change significantly between tests (outbound: F(1, 9) = 1.27, p = 0.289, ηp² = 0.12; return: F(1, 9) = 5.13, p = 0.100, ηp² = 0.36), nor did the number of transitions between regions of the control interface (outbound: F(1, 9) = 0.40, p = 0.542, ηp² = 0.04; return: F(1, 9) = 3.98, p = 0.154, ηp² = 0.31).

Overall, in the second test participants drove the robot along paths of similar length and with a similar number of commands, but in a shorter time and at a higher mean speed, indicating a smoother control of the robot rather than a change in the driving strategy.